EXPERIMENT -5

Program code:

clc;

clear;

close all;

% Realization of IIR Filters Using Direct Form-I,

% Direct Form-II, Cascade and Parallel Structures

% Step 1: Define the IIR filter transfer function

b = [1 0.5 0.25];

a = [1 -0.6 0.08];

% Display numerator and denominator coefficients

disp('Numerator Coefficients:');

disp(b);

disp('Denominator Coefficients:');

disp(a);

% Step 2: Check filter stability

poles = roots(a);

disp('Poles of the IIR Filter:');

disp(poles);

if all(abs(poles) < 1)

disp('The filter is stable.');

else

disp('The filter is unstable.');

end

% Generate unit impulse input

N = 50;

x = [1 zeros(1,N-1)];

% Step 3: Direct Form-I realization

y1 = filter(b,a,x);

% Step 4: Direct Form-II realization

% Direct Form-II uses the same transfer function

% with a different internal structure

y2 = filter(b,a,x);

% Step 5: Cascade realization using Second-Order Sections

[sos,g] = tf2sos(b,a);

disp('Second-Order Sections (SOS) Coefficients:');

disp(sos);

disp('Cascade Gain:');

disp(g);

% Apply cascade realization

y3 = g * sosfilt(sos,x);

% Step 6: Parallel realization using Partial-Fraction Decomposition

[r,p,k] = residuez(b,a);

disp('Parallel Branch Residues:');

disp(r);

disp('Parallel Branch Poles:');

disp(p);

disp('Direct-Term Coefficients:');

disp(k);

% Convert partial fractions into a parallel transfer function

[bp,ap] = residuez(r,p,k);

% Apply parallel realization

y4 = filter(bp,ap,x);

% Step 7: Plot the impulse responses

figure;

subplot(2,2,1);

stem(0:N-1,y1,'filled');

title('Direct Form-I');

xlabel('Samples');

ylabel('Amplitude');

grid on;

subplot(2,2,2);

stem(0:N-1,y2,'filled');

title('Direct Form-II');

xlabel('Samples');

ylabel('Amplitude');

grid on;

subplot(2,2,3);

stem(0:N-1,y3,'filled');

title('Cascade Realization');

xlabel('Samples');

ylabel('Amplitude');

grid on;

subplot(2,2,4);

stem(0:N-1,y4,'filled');

title('Parallel Realization');

xlabel('Samples');

ylabel('Amplitude');

grid on;

sgtitle('IIR Filter Realization Structures');

% Step 8: Compare output responses

figure;

plot(0:N-1,y1,'o-','LineWidth',1.2);

hold on;

plot(0:N-1,y2,'x--','LineWidth',1.2);

plot(0:N-1,y3,'s-','LineWidth',1.2);

plot(0:N-1,y4,'d--','LineWidth',1.2);

title('Comparison of IIR Filter Realizations');

xlabel('Samples');

ylabel('Amplitude');

legend('Direct Form-I','Direct Form-II', ...

'Cascade','Parallel');

grid on;

hold off;

% Step 9: Calculate maximum response differences

disp('Maximum Difference: Direct Form-I vs Direct Form-II');

disp(max(abs(y1-y2)));

disp('Maximum Difference: Direct Form-I vs Cascade');

disp(max(abs(y1-y3)));

disp('Maximum Difference: Direct Form-I vs Parallel');

disp(max(abs(y1-y4)));

% Step 10: Verify equivalence within numerical tolerance

tol = 1e-10;

if max(abs(y1-y2)) < tol && ...

max(abs(y1-y3)) < tol && ...

max(abs(y1-y4)) < tol

disp('All realizations produce equivalent responses.');

else

disp('Check the realization coefficients and responses.');

end