EXPERIMENT-3

PROGRAM CODE:

clc;

clear;

close all;

% IIR Filter Design Using Impulse Invariance and Bilinear Transformation

% Step 1: Filter Specifications

Fs = 1000;

Fc = 100;

N = 3;

% Step 2: Design Analog Butterworth Filter

Wc = 2*pi*Fc;

[b_a, a_a] = butter(N, Wc, 's');

disp('Analog Filter Coefficients:');

disp(b_a);

disp(a_a);

% Step 3: Impulse Invariance Transformation

[b1, a1] = impinvar(b_a, a_a, Fs);

disp('Impulse Invariance Coefficients:');

disp('Numerator:');

disp(b1);

disp('Denominator:');

disp(a1);

% Step 4: Bilinear Transformation

[b2, a2] = bilinear(b_a, a_a, Fs);

disp('Bilinear Transformation Coefficients:');

disp('Numerator:');

disp(b2);

disp('Denominator:');

disp(a2);

% Step 5: Bilinear Transformation with Prewarping

Wp = 2*Fs*tan(pi*Fc/Fs);

[b_ap, a_ap] = butter(N, Wp, 's');

[b3, a3] = bilinear(b_ap, a_ap, Fs);

disp('Bilinear Transformation with Prewarping:');

disp('Numerator:');

disp(b3);

disp('Denominator:');

disp(a3);

% Step 6: Calculate Frequency Responses

[H1, f] = freqz(b1, a1, 2048, Fs);

[H2, ~] = freqz(b2, a2, 2048, Fs);

[H3, ~] = freqz(b3, a3, 2048, Fs);

% Step 7: Plot Magnitude Responses

figure;

plot(f, 20*log10(abs(H1)+eps), 'b', 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(H2)+eps), 'r--', 'LineWidth', 1.5);

plot(f, 20*log10(abs(H3)+eps), 'g-.', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response of IIR Filters');

legend('Impulse Invariance', 'Bilinear Transformation', ...

'Bilinear with Prewarping');

xlim([0 Fs/2]);

% Step 8: Plot Phase Responses

figure;

plot(f, unwrap(angle(H1))*180/pi, 'b', 'LineWidth', 1.5);

hold on;

plot(f, unwrap(angle(H2))*180/pi, 'r--', 'LineWidth', 1.5);

plot(f, unwrap(angle(H3))*180/pi, 'g-.', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Phase Response of IIR Filters');

legend('Impulse Invariance', 'Bilinear Transformation', ...

'Bilinear with Prewarping');

xlim([0 Fs/2]);

% Step 9: Calculate Magnitude at Cutoff Frequency

% Use normalized digital frequency in radians/sample

wc = 2*pi*Fc/Fs;

H1c = freqz(b1, a1, wc);

H2c = freqz(b2, a2, wc);

H3c = freqz(b3, a3, wc);

disp('Magnitude at Cutoff Frequency:');

fprintf('Impulse Invariance: %.2f dB\n', ...

20*log10(abs(H1c)+eps));

fprintf('Bilinear Transformation: %.2f dB\n', ...

20*log10(abs(H2c)+eps));

fprintf('Bilinear with Prewarping: %.2f dB\n', ...

20*log10(abs(H3c)+eps));