EXPERIMENT: 4

PROGRAM CODE:

clc;

clear;

close all;

% Common filter specifications

Fs = 1000;       % Sampling frequency in Hz

Fpass = 100;     % Passband edge in Hz

Fstop = 150;     % Stopband edge in Hz

Rp = 1;          % Passband ripple in dB

Rs = 40;         % Stopband attenuation in dB

Wp = Fpass/(Fs/2);

Ws = Fstop/(Fs/2);

% Butterworth filter

[Nb, Wnb] = buttord(Wp, Ws, Rp, Rs);

[b1, a1] = butter(Nb, Wnb, 'low');

% Chebyshev Type-I filter

[Nc, Wnc] = cheb1ord(Wp, Ws, Rp, Rs);

[b2, a2] = cheby1(Nc, Rp, Wnc, 'low');

% Elliptic filter

[Ne, Wne] = ellipord(Wp, Ws, Rp, Rs);

[b3, a3] = ellip(Ne, Rp, Rs, Wne, 'low');

% Frequency responses

[H1, f] = freqz(b1, a1, 2048, Fs);

H2 = freqz(b2, a2, 2048, Fs);

H3 = freqz(b3, a3, 2048, Fs);

% Magnitude response comparison

figure;

plot(f, 20*log10(abs(H1)+eps), 'b', 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(H2)+eps), 'r', 'LineWidth', 1.5);

plot(f, 20*log10(abs(H3)+eps), 'g', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth, Chebyshev-I and Elliptic IIR Filters');

legend('Butterworth', 'Chebyshev-I', 'Elliptic');

xline(Fpass, '--k', 'Fpass');

xline(Fstop, ':k', 'Fstop');

ylim([-100 5]);

xlim([0 Fs/2]);

% Phase response comparison

figure;

plot(f, unwrap(angle(H1))*180/pi, 'b', 'LineWidth', 1.2);

hold on;

plot(f, unwrap(angle(H2))*180/pi, 'r', 'LineWidth', 1.2);

plot(f, unwrap(angle(H3))*180/pi, 'g', 'LineWidth', 1.2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

title('Phase Responses of IIR Filters');

legend('Butterworth', 'Chebyshev-I', 'Elliptic');

% Display filter orders

fprintf('\nFILTER ORDER COMPARISON\n');

fprintf('Butterworth Order    = %d\n', Nb);

fprintf('Chebyshev-I Order    = %d\n', Nc);

fprintf('Elliptic Order       = %d\n', Ne);

fprintf('Transition Bandwidth = %.2f Hz\n', Fstop-Fpass);

% Display comparative table

Filter = {'Butterworth'; 'Chebyshev-I'; 'Elliptic'};

Order = [Nb; Nc; Ne];

PassbandRipple_dB = [Rp; Rp; Rp];

StopbandAttenuation_dB = [Rs; Rs; Rs];

TransitionBandwidth_Hz = repmat(Fstop-Fpass, 3, 1);

Results = table(Filter, Order, PassbandRipple_dB, ...

StopbandAttenuation_dB, TransitionBandwidth_Hz);

disp(Results);